FIRE FIGHTING ROBOT

The design that's being presented is concentrated on a firefighting robot. Robots are able of performing tasks in a more efficient, cost effective and accurate manner than humans. Fire assumes a significant job in a mortal life still alongside that its dangerous too. A fire fighter work entails detecting and extinguishing fires. It is an amalgamation of fire detection & alarming devices working in tandem with each other with an objective to sense the initial signs of fire and warn.

Problem Statement

Fire and Gas leak incidents poses a significant threat to life and properties a certain condition poses difficult to reach by human and lack of quick reaction may causes serious damage. Therefore, fire- fighting robot are operated in places where firefighters are unfit to work. Besides that, firefighting robots can be used for guarding fire fighters from extreme peril in Petro- chemical, chemical dangerous product, toxin or explored fire accidents.

Proposed Solutions

1. Our system will detect the Fire and Gas leak in real time.

2. Alams the User and Fire station.

3. Shuts off the fire using fire extinguisher or water.

4. It acts a quick reaction which will save many lives and properties

5. It is automated system so no risking of human life.

6. It can reach to narrow spaces.

7. It has more mobility.

Target Audience

1. The primary users of this solution are homeowners and apartments who are looking for primary fire safety system

2.Military and defence.

3.Small fire departments.

4.Forest Areas

Construction

To construct the firefighting robot, the required components are as follows

1.	Arduino Board(uno)

2.USB Cable

2.	GAS sensor

3.	Servo motor

4.	Flame sensor module

5.	L298 Motor drive module

6       Relay

7.	Mini DC Submersible Pump

8.	12V Battery

9.	Switch

10.	GSM module

11.	Buzzer

PIN DIAGRAMS

| L298 N DRIVER PINS | ARDUINO PINS |
| --- | --- |
| IN1 | 8 |
| IN2 | 9 |
| IN3 | 10 |
| IN4 | 11 |

| FLAME SENSOR | ARDUINO PINS |
| --- | --- |
| VCC | 5V |
| GND | GND |
| DO(LEFT) | 4 |
| DO(FORWARD) | 3 |
| DO(RIGHT) | 2 |

| SERVO PINS | ARDUINO PINS |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SIGNAL | 5 |

CODE

#include<Servo.h>

Servo s;

//motors

const int in1 = 8;

const int in2 = 9;

const int in3 = 10;

const int in4 = 11;

const int fl1= 2;

const int fl2= 3;

const int fl3= 4;

const int buz_pin= 12;

const int ser_pin= 5; //servo signal

const int smoke_sens= A0;

//pump inputs

const int water_in1= 6;

const int water_in2= 7;

const int turn_delay=400;

int fl_for=0;

int fl_right=0;

int fl_left=0;

int smoke=0;

int servo_pos=90;

int buzzer_freq=500;

int water_status=0;

void forward(){

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

digitalWrite(in3, LOW);

digitalWrite(in4, HIGH);

}

void back(){

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

digitalWrite(in3, HIGH);

digitalWrite(in4, LOW);

}

void right(){

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

digitalWrite(in3, HIGH);

digitalWrite(in4, LOW);

delay(turn_delay);

}

void left(){

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, HIGH);

delay(turn_delay);

}

void stop(){

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, LOW);

}

void flame_ext(){

digitalWrite(water_in1,LOW);

digitalWrite(water_in2,HIGH);

for(int i=0;i<2;i++){

for(servo_pos=0;servo_pos<180;servo_pos++){

s.write(servo_pos);

delay(15);

}

for(servo_pos=180;servo_pos>0;servo_pos--){

s.write(servo_pos);

delay(15);

}

}

servo_pos=0;

s.write(servo_pos);

digitalWrite(water_in1,HIGH);

digitalWrite(water_in2,LOW);

water_status=0;

}

void setup() {

// put your setup code here, to run once:

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(in3, OUTPUT);

pinMode(in4, OUTPUT);

pinMode(water_in1, OUTPUT);

pinMode(water_in2, OUTPUT);

pinMode(ser_pin, OUTPUT);

pinMode(buz_pin,OUTPUT);

pinMode(smoke_sens,OUTPUT);

pinMode(fl1, INPUT);

pinMode(fl2, INPUT);

pinMode(fl3, INPUT);

s.attach(ser_pin);

Serial.begin(9600);

s.write(servo_pos);

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, LOW);

digitalWrite(water_in1, LOW);

digitalWrite(water_in2, LOW);

}

void loop() {

digitalWrite(buz_pin,LOW);

smoke=analogRead(smoke_sens);

fl_for=digitalRead(fl1);

fl_right=digitalRead(fl2);

fl_left=digitalRead(fl3);

/*Serial.println(fl_right);

Serial.println(fl_left);

Serial.println(fl_for);*/

if(fl_for==1 && fl_right==1 && fl_left==1){

Serial.println("no fire");

forward();

water_status=0;

}

if(fl_for==0 || fl_right==0 || fl_left==0 || smoke>=550)

{

Serial.println("fire");

digitalWrite(buz_pin,HIGH);

//Serial.println(smoke);

Serial.println(fl_for);

if(fl_for==0){

stop();

water_status=1;

}

if(fl_right==0)right();

if(fl_left==0)left();

if(smoke>=550){

right();right();

}

}

if(water_status==1)flame_ext();

}